AI-Powered Object Detection, Segmentation & Real-World Measurement System

Computer Vision Engineering Project Report

# 1.  Project Overview

This system is an end-to-end computer vision pipeline designed to detect, segment, and measure physical objects from images using deep learning and geometric camera calibration techniques.

The system combines:

- Instance segmentation (Mask R-CNN)

- Camera calibration (intrinsic + distortion correction)

- Pixel-to-real-world unit conversion (mm scale estimation)

- Inference pipeline with visualization overlays

- REST API deployment using FastAPI

## Key Capabilities

- Detect objects in images

- Generate pixel-accurate segmentation masks

- Undistort images using camera calibration parameters

- Convert pixel measurements into real-world millimeter values

- Output bounding boxes, masks, and confidence scores

- Serve predictions via API endpoint

# 2.  Setup & Installation Guide

## Environment Setup

- Navigate to the environment.yml present in root directory of project

- conda env create -f environment.yml

- this will create the conda env xai

- Activate conda environment : conda activate xai

- pip install fastapi uvicorn python-multipart install Fastapi for running a simple ui demo

## Run Script

Calibrations: It’s a ipynb so it can be run via notebook ui

- Model: The model is present in train.py

- command: python Model/train.py --data Dataset --epochs 10 --batch_size 2 --lr 0.005

- Visualise the results: python Model/visualisation.py

- Running the inference:

cd D:\Cv_img_segmentation_pipeline

python -m Inference.infer --image "D:\Cv_img_segmentation_pipeline\Dataset\test\WhatsApp Image 2026-06-03 at 3-47-55 PM_jpeg.rf.FdPbK9yTDBb9j7bbYCic.jpeg"

- you can choose any img from the dataset and replace its path

- Pixel-to-MM Conversion: This is integrated into the inference/infer.py and will run with inference using the above command

## Run API Server

uvicorn API.main:app --reload

# 3.  System / Pipeline Architecture

## End-to-End Flow

- Image Input

- Camera Undistortion (Intrinsic Calibration)

- Mask R-CNN Inference

- Post-processing (thresholding + filtering)

- Pixel-to-MM conversion

- Visualization overlay

- API response / output image generation

## Architecture Flow

Input Image

↓

Camera Calibration (K, D)

↓

Undistorted Image

↓

Mask R-CNN Model

↓

Predictions:  Boxes  |  Masks  |  Scores

↓

Post-processing

↓

Pixel → MM Conversion

↓

Annotated Output + JSON Response

# 4.  Camera Calibration Report

## Method Used

OpenCV chessboard-based intrinsic calibration was used to estimate:

- Camera Matrix (K)

- Distortion Coefficients (D)

## Mathematical Model

The pinhole camera model defines corrected image coordinates as:

x_corrected = fx · X + cx

y_corrected = fy · Y + cy

## Distortion Types Corrected

- Radial distortion

- Tangential distortion

## Why Calibration is Required

Raw images contain geometric distortion that leads to the following issues:

- Straight lines appear curved

- Pixel distances are non-uniform

- Measurements become unreliable

Undistortion ensures:

- Linear geometry

- Consistent pixel-to-mm mapping

- Accurate metric estimation

## Output Parameters

- Camera Matrix (K)

- Distortion Coefficients (D)

# 5.  Dataset Description

## Object Class

Single-class dataset: coin and small object instances.

## Collection Method

- Real-world image capture using smartphone camera

- Multiple angles and lighting conditions

- Controlled background variation

## Annotation

- Tool: Roboflow annotation platform

- Format: COCO JSON (images, bounding boxes, segmentation masks)

## Dataset Statistics

- Total Images: 56

- Annotations per image: variable

- Resolution: 900 × 1600

# 6.  Model Training Report

## Model Architecture

Mask R-CNN with ResNet-50 FPN backbone.

## Why Mask R-CNN?

- Performs instance segmentation, not just detection

- Provides pixel-level masks required for measurement

- High accuracy on small object segmentation

- Built-in ROIAlign ensures spatial precision

## Hyperparameters

- Optimizer: SGD / Adam (depending on run)

- Learning Rate: 0.001

- Epochs: 10

- Batch Size: 2–4 (GPU dependent)

- Input normalization: [0, 1] scaling

## Evaluation Metrics

- mAP@0.5

- mAP@0.5:0.95

- Precision & Recall

- IoU-based evaluation

## Final Training Performance

| Epoch | mAP@0.5:0.95 | Train Loss |
| --- | --- | --- |
| 2 | 0.894 | 0.2079 |
| 3 | 0.919 | 0.1280 |
| 4 | 0.971 | 0.1141 |
| 6 | 0.985 | 0.0898 |
| 9 | 0.993 | 0.0774 |
| 10 | 0.991 | 0.0738 |

Best model achieved mAP@0.5:0.95 ≈ 0.99, with strong convergence from epoch 6 onwards.

# 7.  Measurement Methodology

The objective is to convert pixel-based segmentation masks into real-world measurements in millimetres.

## 7.1  Pixel-to-MM Conversion

A scale factor is defined as:

Scale = Real Object Dimension (mm) / Pixel Dimension

Then:

Width_mm  = Width_px  × Scale

Height_mm = Height_px × Scale

## 7.2  Mask-Based Measurement

Instead of bounding boxes, segmentation masks are used to extract object contours and compute:

- Pixel width

- Pixel height

This approach increases accuracy significantly for irregular shapes.

## 7.3 Calibration Dependency

All measurements depend on undistorted images. Issues with raw images include:

- Lens distortion shifts pixel geometry

- Non-linear scaling across the image plane

- Edge warping causes incorrect width/height estimates

Solution — undistortion is applied prior to inference:

image = cv2.undistort(image, K, D)

## 7.4 Accuracy Validation

### Ground Truth Method

- Physical measurement using caliper/ruler

- 10+ samples measured

### Metrics

Mean Absolute Error (MAE):

MAE = (1/n) · Σ |y_pred − y_true|

Mean Percentage Error (MPE):

MPE = (1/n) · Σ (|y_pred − y_true| / y_true) × 100

## 7.5 Results

| Sample | True (mm) | Predicted (mm) | Error (mm) |
| --- | --- | --- | --- |
| 1 | 25.0 | 24.7 | 0.3 |
| 2 | 10.0 | 10.2 | 0.2 |
| 3 | 18.0 | 18.4 | 0.4 |

# 8.  API / Module Documentation

## FastAPI Endpoint

POST /predict

### Input

- Image file upload

### Output

{

"results": [

{

"confidence": 0.99,

"width_mm": 23.25,

"height_mm": 10.5

}

],

"output_image": "static/outputs/xxx.jpg"

}

## Processing Pipeline

- Receive image

- Undistort using calibration parameters

- Run Mask R-CNN inference

- Extract masks and confidence scores

- Convert pixels → mm

- Return JSON response and annotated image

# 9.  Design Decisions

## Mask R-CNN

- Instance-level segmentation

- High precision masks

- Direct compatibility with COCO format

## OpenCV Calibration

- Industry-standard accuracy

- Fast computation

- Easy integration

## COCO Dataset Format

- Native Mask R-CNN compatibility

- Standard evaluation pipeline (pycocotools)

## FastAPI

- Lightweight deployment

- Fast inference serving

- Easy integration with frontend UI

# 10.  Assumptions & Limitations

## Assumptions

- Camera intrinsic parameters remain constant

- Object lies on a planar surface

- Lighting conditions are stable

## Limitations

- Performance depends on calibration accuracy

- Small dataset (56 images) limits generalization

- Single-class detection only

- Measurement accuracy reduces for occluded objects

- No depth estimation (2D limitation)

# 11.  End-to-End Usage Guide

## Step 1 — Train Model

python Model/train.py --data Dataset --epochs 10 --batch_size 2 --lr 0.005

## Step 2 — Run Inference

d D:\Cv_img_segmentation_pipeline

python -m Inference.infer –image “Path to img”

## Step 3 — Launch API

uvicorn API.main:app --reload

## Step 4 — Output

- Segmentation overlay image

- JSON response containing: confidence, width (mm), height (mm)

# 12.  Final Summary

This project demonstrates a complete computer vision pipeline that integrates the following components:

- Deep learning via Mask R-CNN

- Camera geometry through calibration and undistortion

- Metric reconstruction using pixel-to-mm conversion

- Production deployment via FastAPI

The system successfully bridges the gap between visual perception and real-world measurement, enabling accurate object-scale estimation from monocular images.